Field descriptions with units for the ASCII archive provided:

| Field | Name | Description | Units |
| --- | --- | --- | --- |
| 1 | GPS_UTCSSM | UTC Seconds After Midnight | s |
| 2 | lat_diff | Differentially Corrected Latitude - WGS84 Datum | deg.deg |
| 3 | long_diff | Differentially Corrected Longitude - WGS84 Datum | deg.deg |
| 4 | x_wgs84 | Easting (X) - UTM Zone 18 North WGS84 Datum | m |
| 5 | y_wgs84 | Northing (Y) - UTM Zone 18 North WGS84 Datum | m |
| 6 | gpsz_diff | Differentially Corrected GPS Elevation (Above Mean Sea Level) | m |
| 7 | srtm | Digital Topographic Model (from SRTM, Above MSL) | m |
| 8 | alt_radar | Radar Altimeter | m |
| 9 | calc_height | Differentially Corrected Calculated Height Above Ground | m |
| 10 | Pressure | Outside Air Pressure | hPa / mbar |
| 11 | Temperature | Outside Air Temperature | deg C |
| 12 | FID | FASDAS Fiducial Counter | s |
| 13 | UtcTime | UTC Time | hh.hhhhh |
| 14 | Cosmic_R | Raw Cosmic Window Counts (>3 MeV) | counts |
| 15 | Kcnt_R | Raw Potassium Window Counts (1370 - 1570 keV) | counts |
| 16 | Tcnt_R | Raw Thorium Window Count (2410 - 2810 keV) | counts |
| 17 | Totc_R | Raw Total Count Window (410 - 2810 keV) | counts |
| 18 | Ucnt_R | Raw Uranium Window Counts (1660 - 1860 keV) | counts |
| 19 | Uup_R | Raw Upward Uranium Window Counts (1660 - 1860 keV) | counts |
| 20 | LiveTime | Spectrometer Live Time | ms |
| 21-276 | SpectraD[CH] | 256 Channel Downward Spectrum Array CH0 = Sample Time, CH 255 = Cosmic (>3MeV) CH1-254 are linear fit to 662 keV at CH55.5 and 2615 keV at 218.5 if automatic gain control is ideal | counts |
| 277-532 | SpectraU[CH] | 256 Channel Upward Spectrum Array CH0 to CH255 fit to line as above | counts |

Note – Null values in the ASCII archive are displayed as *